Home Page

Product Overview

Contact / Links / Partners

< Back to Product List

A reliable partner in gas analysis & monitoring

ETG Risorse e Tecnologia

CALG-100

Permeation Calibrator

The calibrator  ETG CALG100  has been designed to provide a high level of calibration for the use of analyzers HC and BTX (ppb level). The use of the calibrator is very simple, thanks to the high automation of its components. In addition, the instrument is equipped with two USB ports which can be controlled directly from a PC connected to it the solenoid calibration (zero and span) and the direct flow to the oven, that needs to set the value of the  gas dilution (controlled by a MFC). On the front panel you can set a temperature value that will remain stable over time. The calibrator is mounted in a drawer rack size 3U. The calibrator includes a computerized management software for its.

The calibrator is equipped with two solenoid calibration: zero and span. using software supplied with, the user can, depending on the setting you must set (to use the software refer to the section of the manual) to activate one or the other solenoid. If, for example, we are conducting a span calibration, after setting the desired value, the system (by a formula) will instruct the MFC flow to be applied.

Once inserted into the permeation tube oven, and set its operating temperature, the system, for example, if you are calibrating the span, will set a value stream that will serve to dilute the gas calculated from the concentration desired that the user has entered in the software. In the meantime, checks will be performed by a system that maintains the temperature and pressure. all other components (such as relays, switches, transformers, etc ...) are only the automation of the instrument.

Microprocessor-based temperature controller

featuring proportional heater control and digital

display of set point and oven temperatures

Precision dilution and optimal flow

thanks to Mass Flow Controller

Electrovalves for automatic selection of

Zero/Span/Sample

Software available

Digital display of temperature and set point

PT100  temperature sensor

Front panel mounted pressure regulator,

pressure  guage, temperature controller

and oven inlet

Operating Range:

1 deg. C above ambient to 60 deg. C,

Unit Operating Temperature:

5 deg. C to 45 deg. C

Warm Up:

Less than 30 minutes

230 VAC, 50/60 Hz

Unit Size 5.25” (13.3 cm) x 19”

(48.3 cm) W x 15” (38.1cm) D

Weight 18 lbs (8.2 kg)

Rack Mount Kit 3U

Weight 18 lbs (8.2 kg)